LAB 1-1 SETTING UP YOUR BREADBOARD AND CONNECTING DEVICES TO IT

| Description | In this lab, you will setup your breadboard to be connected to the Raspberry Pi using the Adafruit T CobblerPlus Kit and connect an LED light and Push Button switch to it and write Python programs to make these devices work. |
| --- | --- |
| What you will learn | - How to setup your breadboard to be connected to the Raspberry Pi - How to connect an LED light and a Push Button switch to the Pi using the breadboard. - How to write Python programs to make these devices work |
| What you need | - Adafruit T Cobbler Plus and Breakout Cable - Breadboard and Jumper wires - LED light (red, green, yellow) - Push Button switch |
| Duration | 45 minutes |

Connecting your Cobbler Kit to your Breadboard

- Your breadboard is connected internally as follows:

- Connect your Cobbler Kit to your breadboard based on the diagram below:

- It is conventional to use the horizontal rows below and above to connect to Gnd and 5V respectively.

Connecting and programming LEDs

- Connect an LED to your breadboard and Cobbler kit as follows:

  - Connect Pi Cobbler #12 to Positive end of LED (LONG end)

  - Connect Pi Cobbler GND to Negative end of LED (SHORT end)

- Connecting to VNC Viewer

  - Turn on your Pi and wait a few mins

  - Connect to SSID from your Raspberry Pi, eg pi723_1)

  - Wireless password : raspberry

  - Launch VNC Viewer

  - Connect to 192.158.1.8

  - Username: pi

  - Password: raspberry

- Create a new Python script named ledlights.py and save it in the /home/pi directory

Codes are available on your Pi Desktop/P1Codes

import RPi.GPIO as GPIO

import time

from time import sleep

#dismisses error messages

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

#Creating a blinking method

def blinking(pin):

#turns on LED for 2 seconds

GPIO.output(pin, GPIO.HIGH)

time.sleep(2)

#turns off LED for 2 seconds

GPIO.output(pin, GPIO.LOW)

time.sleep(2)

return

#states where the LED pin is

LED_PIN = 12

GPIO.setup(LED_PIN, GPIO.OUT)

#Calling the Blinking method over a span of 20 seconds

for i in range (0, 20):

blinking(LED_PIN)

GPIO.cleanup()

Notes on this program

- import RPi.GPIO as GPIO allows us to make use of the Pi’s GPIO pins

- GPIO.setwarnings(False) prevents the program from displaying unimportant warning messages during code execution

- GPIO.BCM indicates that you refer to the Pi’s GPIO pins by the Broadcom SOC channel number

- LED_PIN = 12 states that the LED is using Pin 12 on the Cobbler Kit

- GPIO.output(pin, GPIO.HIGH) turns on the LED, while GPIO.output(pin, GPIO.LOW) turns the LED off

Connecting Push Button to Illuminate LEDs

- Add the push button to the existing connections on the breadboard:

  - Connect Pi Cobbler #13 to left end of push button

  - Connect GND from Pi Cobbler to right end of push button

- Create a Python script names pushbtnled.py and save it in the /home/pi directory

import RPi.GPIO as GPIO

import time

from time import sleep

#dismisses error messages

GPIO.setwarnings(False)

GPIO.setmode(GPIO.BCM)

#states where the LED pin is

LED_PIN = 12

GPIO.setup(LED_PIN, GPIO.OUT)

#states where the Push Button pin is

PUSH_BUTTON = 13

GPIO.setup(PUSH_BUTTON, GPIO.IN, pull_up_down=GPIO.PUD_UP)

while True:

if GPIO.input(PUSH_BUTTON) == False:

GPIO.output(LED_PIN, GPIO.HIGH)

time.sleep(1)

GPIO.output(LED_PIN, GPIO.LOW)

GPIO.cleanup()